COMPUTER NETWORK

Computer Network

A network set up by connecting two or more computers and other supporting hardware  devices through communication channels is called a computer network. It enables computers to  communicate with each other and to share commands, data, etc., including the hardware and  software resources. In the case of computer network technology, there are several types of  networks that vary from simple to complex level.

Uses of Computer Network:

o Resource sharing: Resource sharing is the sharing of resources such as programs,  printers, and data among the users on the network without the requirement of the  physical location of the resource and user.

o Server-Client model: Computer networking is used in the server-client model. A server  is a central computer used to store the information and maintained by the system  administrator. Clients are the machines used to access the information stored in the  server remotely.

o Communication medium: Computer network behaves as a communication medium  among the users. For example, a company contains more than one computer has an  email system which the employees use for daily communication.

o E-commerce: Computer network is also important in businesses. We can do the business  over the internet. For example, amazon.com is doing their business over the internet,  i.e., they are doing their business over the internet.

Components Of Computer Network:

Major components of a computer network are:

NIC(National interface card)

NIC is a device that helps the computer to communicate with another device. The network  interface card contains the hardware addresses, the data-link layer protocol use this address to  identify the system on the network so that it transfers the data to the correct destination. There are two types of NIC: wireless NIC and wired NIC.

o Wireless NIC: All the modern laptops use the wireless NIC. In Wireless NIC, a connection  is made using the antenna that employs the radio wave technology.

o Wired NIC: Cables use the wired NIC to transfer the data over the medium.

Hub

Hub is a central device that splits the network connection into multiple devices. When  computer requests for information from a computer, it sends the request to the Hub. Hub  distributes this request to all the interconnected computers.

Switches

Switch is a networking device that groups all the devices over the network to transfer the  data to another device. A switch is better than Hub as it does not broadcast the message over the  network, i.e., it sends the message to the device for which it belongs to. Therefore, we can say  that switch sends the message directly from source to the destination.

Router

Router is a device that connects the LAN to the internet. The router is mainly used to  connect the distinct networks or connect the internet to multiple computers.

Modem

Modem connects the computer to the internet over the existing telephone line. A modem  is not integrated with the computer motherboard. A modem is a separate part on the PC slot  found on the motherboard.

Cables and Connectors

Cable is a transmission media used for transmitting a signal. There are three types of  cables used in transmission:

o Twisted pair cable

o Coaxial cable

o Fibre-optic cable

Features Of Computer network

A list Of Computer network features is given below.

o Communication speed: Network provides us to communicate over the network in a  fast and efficient manner. For example, we can do video conferencing, email  messaging, etc. over the internet. Therefore, the computer network is a great way to  share our knowledge and ideas.

o File sharing: File sharing is one of the major advantage of the computer network.  Computer network provides us to share the files with each other.

o Back up and Roll back is easy: Since the files are stored in the main server which is  centrally located. Therefore, it is easy to take the back up from the main server. o Software and Hardware sharing: We can install the applications on the main server,  therefore, the user can access the applications centrally. So, we do not need to install  the software on every machine. Similarly, hardware can also be shared.

o Security: Network allows the security by ensuring that the user has the right to access  the certain files and applications.

o Scalability: Scalability means that we can add the new components on the network.  Network must be scalable so that we can extend the network by adding new devices.  But, it decreases the speed of the connection and data of the transmission speed also  decreases, this increases the chances of error occurring. This problem can be  overcome by using the routing or switching devices.

o Reliability: Computer network can use the alternative source for the data  communication in case of any hardware failure.

Computer Network Architecture

Computer Network Architecture is defined as the physical and logical design of the  software, hardware, protocols, and media of the transmission of data. Simply we can say that  how computers are organized and how tasks are allocated to the computer. The two types of network architectures are used:

o Peer-To-Peer network

o Client/Server network

Peer-To-Peer network

o Peer-To-Peer network is a network in

which all the computers are linked

together with equal privilege and

responsibilities for processing the data.

o Peer-To-Peer network is useful for small

environments, usually up to 10

computers.

o Peer-To-Peer network has no dedicated server.

o Special permissions are assigned to each computer for sharing the resources, but this can  lead to a problem if the computer with the resource is down.

Advantages Of Peer-To-Peer Network:

o It is less costly as it does not contain any dedicated server.

o If one computer stops working but, other computers will not stop working. o It is easy to set up and maintain as each computer manages itself.

Disadvantages Of Peer-To-Peer Network:

o In the case of Peer-To-Peer network, it does not contain the centralized system .  Therefore, it cannot back up the data as the data is different in different locations. o It has a security issue as the device is managed itself.

Client/Server Network

o Client/Server network is a network model designed for the end users called clients, to  access the resources such as songs, video, etc.

from a central computer known as Server.

o The central controller is known as a server while

all other computers in the network are

called clients.

o A server performs all the major operations such as

security and network management.

o A server is responsible for managing all the

resources such as files, directories, printer, etc.

o All the clients communicate with each other through a server. For example, if client1  wants to send some data to client 2, then it first sends the request to the server for the  permission. The server sends the response to the client 1 to initiate its communication  with the client 2.

Advantages Of Client/Server network:

o A Client/Server network contains the centralized system. Therefore we can back  up the data easily.

o A Client/Server network has a dedicated server that improves the overall  performance of the whole system.

o Security is better in Client/Server network as a single server administers the shared  resources.

o It also increases the speed of the sharing resources.

Disadvantages Of Client/Server network:

o Client/Server network is expensive as it requires the server with large memory. o A server has a Network Operating System(NOS) to provide the resources to the  clients, but the cost of NOS is very high.

o It requires a dedicated network administrator to manage all the resources.

Computer Network Types

A computer network is a group of computers linked to each other that enables the  computer to communicate with another computer and share their resources, data, and  applications. A computer network can be categorized by their size. A computer network is mainly  of four types:

o Local Area Network (LAN)

o Personal Area Network (PAN)

o Metropolitan Area Network (MAN)

o Wide Area Network (WAN)

Local Area Network (LAN):

As the name suggests, the local area network is a computer network that operates in a  small area, i.e., it connects computers in a small geographical area like within an office, company,  school, or any other organization. So, it exists within a

specific area, e.g. home network, office network,

school network, etc.

A local area network may be a wired or wireless

network or a combination of both. The devices in a

LAN are generally connected using an Ethernet cable,

which offers an interface to connect multiple devices

like router, switches, and computers. For example,

using a single router, few Ethernet cables, and

computers, you can create a LAN at your home, office, etc. In this network, one computer may  act as a server and other computers, which are part of the network, may serve as clients.

Benefits of LAN:

o It offers a higher operating speed than WAN and MAN.

o It is less expensive and easy to install and maintain.

o It perfectly fulfills the requirement of a specific organization, such as an office,  school, etc.

o It can be wired or wireless or a combination of both.

o It is more secure than other networks as it is a small set up that can be easily taken  care of.

Primary Functions of LAN:

o Sharing of files: It allows you to share or transfer files from one computer to another  computer within the LAN. For example, in a bank, it can be used to send a file with the  details of transactions of a customer from the server to clients.

o Sharing of printers: It also allows shared access to a printer, file servers, etc. For example,  ten computers that are connected through LAN can use a single printer, file server, fax  machine, etc.

o Sharing of Computational capabilities: It allows the clients to access to the  computational power of a server, e.g., an application server as some applications which  run on clients in a LAN may require higher computational capabilities.

o Mail and message related services: It allows sending and receiving mails between  computers of a LAN. You are required to have a mail server for this.

o Database services: It also allows storing and retrieving data with the help of a database  server.

Personal Area Network (PAN)

o Personal Area Network is a network arranged within an individual person, typically within  a range of 10 meters.

o Personal Area Network is used for

connecting the computer devices of

personal use is known as Personal

Area Network.

o Thomas Zimmerman was the first

research scientist to bring the idea of

the Personal Area Network.

o Personal Area Network covers an

area of 30 feet.

o Personal computer devices that are used to develop the personal area network are the  laptop, mobile phones, media player and play stations.

There are two types of Personal Area Network:

o Wired Personal Area Network: Wireless Personal Area Network is developed by  simply using wireless technologies such as WiFi, Bluetooth. It is a low range network. o Wireless Personal Area Network: Wired Personal Area Network is created by using  the USB.

Examples Of Personal Area Network:

o Body Area Network: Body Area Network is a network that moves with a person. For  example, a mobile network moves with a person. Suppose a person establishes a network  connection and then creates a connection with another device to share the information.

o Offline Network: An offline network can be created inside the home, so it is also known  as a home network. A home network is designed to integrate the devices such as printers,  computer, television but they are not connected to the internet.

o Small Home Office: It is used to connect a variety of devices to the internet and to a  corporate network using a VPN

Metropolitan Area Network (MAN):

MAN is a high-speed network that spreads over a large geographical area such as a metro  city or town. It is set up by connecting the local area networks using routers and local telephone  exchange lines. It can be operated by a

private company, or it may be a service

provided by a company such as a local

telephone company.

MAN is ideal for the people of a

relatively large area who want to share

data or information. It provides fast

communication via high-speed carriers or

transmission media such as copper, fiber

optics, and microwaves. The commonly used protocols for MAN are X.25, Frame Relay,  Asynchronous Transfer Mode (ATM), xDSL (Digital Subscriber Line), ISDN (Integrated Services  Digital Network), ADSL (Asymmetrical Digital Subscriber Line), and more.

The area covered by MAN is larger than the LAN but smaller than a WAN. Its network  ranges from 5 to 50 km. Furthermore, it also provides uplinks for connecting LANs to WANs and  the internet. An organization can use a MAN to connect all of its LANs located at its different  offices across the city.

Uses Of Metropolitan Area Network:

o MAN is used in communication between the banks in a city.

o It can be used in an Airline Reservation.

o It can be used in a college within a city.

o It can also be used for communication in the military.

Examples of MAN:

o Cable TV Network

o Telephone service provides that provide high-speed DSL lines

o IEEE 802.16 or WiMAX

o Connected fire stations in a city

o Connected branches of a school in a city

Advantages of MAN:

o Less Expensive: It is less expensive to set up a MAN and to connect it to a WAN. o High Speed: The speed of data transfer is more than WAN.

o Local Emails: It can send local emails fast.

o Access to the Internet: It allows you to share your internet connection, and thus multiple  users can have access to high-speed internet.

o Easy to set up: You can easily set up a MAN by connecting multiple LANs. o High Security: It is more secure than WAN.

Wide Area Network (WAN):

WAN extends over a large geographical area. It is not confined within an office, school,  city, or town and is mainly set up by telephone lines, fiber optic, or satellite links. It is mostly used  by big organizations like banks and multinational companies to communicate with their branches  and customers across the world.

Although it is structurally similar to

MAN, it is different from MAN in

terms of its range, e.g., MAN

covers up to 50 Kms, whereas

WAM covers distances larger than

50 Km, e.g., 1000km or more.

A WAN works by using

TCP/IP protocol in combination

with networking devices such as

switches, routers, firewalls, and

modems. It does not connect individual computers; rather, they are designed to link small

networks like LANs and MANs to create a large network. The internet is considered the largest  WAN in the world as it connects various LANs and MANs through ISPs.

The computers are connected to the wide area network through public networks, such as  telephone systems, leased lines or satellites. The users of a WAN do not own the network as it is  a large setup connecting the remote computer systems. However, they are required to subscribe  to a service provided by a telecommunication provider to use this network.

Advantages of a WAN:

o Geographical area: A Wide Area Network provides a large geographical area. Suppose if  the branch of our office is in a different city then we can connect with them through WAN.  The internet provides a leased line through which we can connect with another branch.

o Centralized data: In case of WAN network, data is centralized. Therefore, we do not need  to buy the emails, files or back up servers.

o Get updated files: Software companies work on the live server. Therefore, the  programmers get the updated files within seconds.

o Exchange messages: In a WAN network, messages are transmitted fast. The web  application like Facebook, Whatsapp, Skype allows you to communicate with friends. o Sharing of software and resources: In WAN network, we can share the software and  other resources like a hard drive, RAM.

o Global business: We can do the business over the internet globally.

o High bandwidth: If we use the leased lines for our company then this gives the high  bandwidth. The high bandwidth increases the data transfer rate which in turn increases  the productivity of our company.

Disadvantages of Wide Area Network:

The following are the disadvantages of the Wide Area Network:

o Security issue: A WAN network has more security issues as compared to LAN and MAN  network as all the technologies are combined together that creates the security problem. o Needs Firewall & antivirus software: The data is transferred on the internet which can be  changed or hacked by the hackers, so the firewall needs to be used. Some people can

inject the virus in our system so antivirus is needed to protect from such a virus. o High Setup cost: An installation cost of the WAN network is high as it involves the  purchasing of routers, switches.

o Troubleshooting problems: It covers a large area so fixing the problem is difficult.

Examples of WAN:

o Mobile Broadband: A 4G network is widely used across a region or country. o Last mile: A telecom company is used to provide the internet services to the customers in  hundreds of cities by connecting their home with fiber.

o Private network: A bank provides a private network that connects the 44 offices. This  network is made by using the telephone leased line provided by the telecom company.

Types Of Internetwork:

1. Extranet: An extranet is a communication network based on the internet protocol such  as Transmission Control protocol and internet protocol. It is used for information  sharing. The access to the extranet is restricted to only those users who have login  credentials. An extranet is the lowest level of internetworking. It can be categorized

as MAN, WAN or other computer networks. An extranet cannot have a single LAN,  atleast it must have one connection to the external network.

2. Intranet: An intranet is a private network based on the internet protocol such  as Transmission Control protocol and internet protocol. An intranet belongs to an  organization which is only accessible by the organization's employee or members. The  main aim of the intranet is to share the information and resources among the organization  employees. An intranet provides the facility to work in groups and for teleconferences.

Intranet advantages:

o Communication: It provides a cheap and easy communication. An employee of the  organization can communicate with another employee through email, chat. o Time-saving: Information on the intranet is shared in real time, so it is time-saving. o Collaboration: Collaboration is one of the most important advantage of the intranet. The  information is distributed among the employees of the organization and can only be  accessed by the authorized user.

o Platform independency: It is a neutral architecture as the computer can be connected to  another device with different architecture.

o Cost effective: People can see the data and documents by using the browser and distributes the duplicate copies over the intranet. This leads to a reduction in the cost.

What is Topology?

Topology defines the structure of the network of how all the components are  interconnected to each other. There are two types of topology: physical and logical topology. Physical topology is the geometric representation of all the nodes in a network.

Types of Network Topology

1. Bus Topology

2. Ring Topology

3. Tree Topology

4. Star Topology

5. Mesh Topology

6. Hybrid Topology

Bus Topology

o The bus topology is designed in such a way that all the stations are connected  through a single cable known as a

backbone cable.

o Each node is either connected to the

backbone cable by drop cable or directly

connected to the backbone cable.

o When a node wants to send a message

over the network, it puts a message over

the network. All the stations available in

the network will receive the message

whether it has been addressed or not.

o The bus topology is mainly used in 802.3 (ethernet) and 802.4 standard networks.

o The configuration of a bus topology is quite simpler as compared to other  topologies.

o The backbone cable is considered as a "single lane" through which the message is  broadcast to all the stations.

o The most common access method of the bus topologies is CSMA (Carrier Sense  Multiple Access).

CSMA: It is a media access control used to control the data flow so that data integrity is  maintained, i.e., the packets do not get lost. There are two alternative ways of handling  the problems that occur when two nodes send the messages simultaneously.

o CSMA CD: CSMA CD (Collision detection) is an access method used to detect the  collision. Once the collision is detected, the sender will stop transmitting the data.  Therefore, it works on "recovery after the collision".

o CSMA CA: CSMA CA (Collision Avoidance) is an access method used to avoid the  collision by checking whether the transmission media is busy or not. If busy, then  the sender waits until the media becomes idle. This technique effectively reduces  the possibility of the collision. It does not work on "recovery after the collision".

Advantages of Bus topology:

o Low-cost cable: In bus topology, nodes are directly connected to the cable  without passing through a hub. Therefore, the initial cost of installation is low. o Moderate data speeds: Coaxial or twisted pair cables are mainly used in bus based networks that support upto 10 Mbps.

o Familiar technology: Bus topology is a familiar technology as the installation and  troubleshooting techniques are well known, and hardware components are easily  available.

o Limited failure: A failure in one node will not have any effect on other nodes.

Disadvantages of Bus topology:

o Extensive cabling: A bus topology is quite simpler, but still it requires a lot of  cabling.

o Difficult troubleshooting: It requires specialized test equipment to determine the  cable faults. If any fault occurs in the cable, then it would disrupt the  communication for all the nodes.

o Signal interference: If two nodes send the messages simultaneously, then the signals of both the nodes collide with each other.

o Reconfiguration difficult: Adding new devices to the network would slow down  the network.

o Attenuation: Attenuation is a loss of signal leads to communication issues.  Repeaters are used to regenerate the signal.

Ring Topology

o Ring topology is like a bus topology, but

with connected ends.

o The node that receives the message from

the previous computer will retransmit to the

next node.

o The data flows in one direction, i.e., it is unidirectional.

o The data flows in a single loop continuously known as an endless loop. o It has no terminated ends, i.e., each node is connected to other node and having  no termination point.

o The data in a ring topology flow in a clockwise direction.

o The most common access method of the ring topology is token passing. o Token passing: It is a network access method in which token is passed from  one node to another node.

o Token: It is a frame that circulates around the network.

Working of Token passing

o A token moves around the network, and it is passed from computer to computer  until it reaches the destination.

o The sender modifies the token by putting the address along with the data. o The data is passed from one device to another device until the destination address  matches. Once the token received by the destination device, then it sends the  acknowledgment to the sender.

o In a ring topology, a token is used as a carrier.

Advantages of Ring topology:

o Network Management: Faulty devices can be removed from the network without  bringing the network down.

o Product availability: Many hardware and software tools for network operation and  monitoring are available.

o Cost: Twisted pair cabling is inexpensive and easily available. Therefore, the  installation cost is very low.

o Reliable: It is a more reliable network because the communication system is not  dependent on the single host computer.

Disadvantages of Ring topology:

o Difficult troubleshooting: It requires specialized test equipment to determine the  cable faults. If any fault occurs in the cable, then it would disrupt the  communication for all the nodes.

o Failure: The breakdown in one station leads to the failure of the overall network. o Reconfiguration difficult: Adding new devices to the network would slow down the  network.

o Delay: Communication delay is directly proportional to the number of nodes.  Adding new devices increases the communication delay.

Star Topology

o Star topology is an arrangement of the

network in which every node is connected

to the central hub, switch or a central

computer.

o The central computer is known as a server,

and the peripheral devices attached to the

server are known as clients.

o Coaxial cable or RJ-45 cables are used to connect the computers. o Hubs or Switches are mainly used as connection devices in a physical star  topology.

o Star topology is the most popular topology in network implementation.

Advantages of Star topology

o Efficient troubleshooting: Troubleshooting is quite efficient in a star topology as  compared to bus topology. In a bus topology, the manager has to inspect the  kilometers of cable. In a star topology, all the stations are connected to the  centralized network. Therefore, the network administrator has to go to the single  station to troubleshoot the problem.

o Network control: Complex network control features can be easily implemented in  the star topology. Any changes made in the star topology are automatically  accommodated.

o Limited failure: As each station is connected to the central hub with its own cable,  therefore failure in one cable will not affect the entire network.

o Familiar technology: Star topology is a familiar technology as its tools are cost effective.

o Easily expandable: It is easily expandable as new stations can be added to the open  ports on the hub.

o Cost effective: Star topology networks are cost-effective as it uses inexpensive  coaxial cable.

o High data speeds: It supports a bandwidth of approx 100Mbps. Ethernet 100BaseT  is one of the most popular Star topology networks.

Disadvantages of Star topology

o A Central point of failure: If the central hub or switch goes down, then all the  connected nodes will not be able to communicate with each other.

o Cable: Sometimes cable routing becomes difficult when a significant amount of  routing is required.

Tree topology

o Tree topology combines the characteristics of bus topology and star topology. o A tree topology is a type of structure in which all the computers are connected with  each other in hierarchical fashion.

o The top-most node in tree topology

is known as a root node, and all other

nodes are the descendants of the

root node.

o There is only one path exists between

two nodes for the data transmission.

Thus, it forms a parent-child

hierarchy.

Advantages of Tree topology

o Support for broadband transmission: Tree topology is mainly used to provide  broadband transmission, i.e., signals are sent over long distances without being  attenuated.

o Easily expandable: We can add the new device to the existing network. Therefore,  we can say that tree topology is easily expandable.

o Easily manageable: In tree topology, the whole network is divided into segments  known as star networks which can be easily managed and maintained. o Error detection: Error detection and error correction are very easy in a tree  topology.

o Limited failure: The breakdown in one station does not affect the entire network. o Point-to-point wiring: It has point-to-point wiring for individual segments.

Disadvantages of Tree topology

o Difficult troubleshooting: If any fault occurs in the node, then it becomes difficult  to troubleshoot the problem.

o High cost: Devices required for broadband transmission are very costly. o Failure: A tree topology mainly relies on main bus cable and failure in main bus  cable will damage the overall network.

o Reconfiguration difficult: If new devices are added, then it becomes difficult to  reconfigure.

Mesh topology

o Mesh technology is an arrangement of the network in which computers are  interconnected with each other through various redundant connections. o There are multiple paths from one

computer to another computer.

o It does not contain the switch, hub or

any central computer which acts as a

central point of communication.

o The Internet is an example of the mesh

topology.

o Mesh topology is mainly used for WAN

implementations where communication

failures are a critical concern.

o Mesh topology is mainly used for wireless networks.

o Mesh topology can be formed by using the formula: Number of cables = (n*(n-1))/2;

Where n is the number of nodes that represents the network.

Mesh topology is divided into two categories:

o Fully connected mesh topology: In a full mesh topology, each computer is  connected to all the computers available in the network.

o Partially connected mesh topology: In a partial mesh topology, not all but certain  computers are connected to those computers with which they communicate  frequently.

Advantages of Mesh topology:

o Reliable: The mesh topology networks are very reliable as if any link breakdown  will not affect the communication between connected computers.

o Fast Communication: Communication is very fast between the nodes. o Easier Reconfiguration: Adding new devices would not disrupt the  communication between other devices.

Disadvantages of Mesh topology

o Cost: A mesh topology contains a large number of connected devices such as a  router and more transmission media than other topologies.

o Management: Mesh topology networks are very large and very difficult to  maintain and manage. If the network is not monitored carefully, then the  communication link failure goes undetected.

o Efficiency: In this topology, redundant connections are high that reduces the  efficiency of the network.

Hybrid Topology

o The combination of various different topologies is known as Hybrid topology. o A Hybrid topology is a connection between different links and nodes to transfer  the data.

o When two or more different topologies are combined together is termed as Hybrid  topology and if similar

topologies are

connected with each

other will not result in

Hybrid topology. For

example, if there exist a

ring topology in one

branch of ICICI bank

and bus topology in

another branch of ICICI

bank, connecting these

two topologies will

result in Hybrid

topology.

Advantages of Hybrid Topology

o Reliable: If a fault occurs in any part of the network will not affect the functioning  of the rest of the network.

o Scalable: Size of the network can be easily expanded by adding new devices  without affecting the functionality of the existing network.

o Flexible: This topology is very flexible as it can be designed according to the  requirements of the organization.

o Effective: Hybrid topology is very effective as it can be designed in such a way that  the strength of the network is maximized and weakness of the network is  minimized.

Disadvantages of Hybrid topology

o Complex design: The major drawback of the Hybrid topology is the design of the  Hybrid network. It is very difficult to design the architecture of the Hybrid network. o Costly Hub: The Hubs used in the Hybrid topology are very expensive as these  hubs are different from usual Hubs used in other topologies.

o Costly infrastructure: The infrastructure cost is very high as a hybrid network  requires a lot of cabling, network devices, etc.