Computer Memory

The computer memory holds the data and instructions needed to process raw data and  produce output. The computer memory is divided into large number of small parts known as cells.  Each cell has a unique address which varies from 0 to memory size minus one.

Computer memory is of two types: Volatile (RAM) and Non-volatile (ROM). The secondary  memory (hard disk) is referred as storage not memory.

But, if we categorize memory on behalf of space or location, it is of four types: o Register memory

o Cache memory

o Primary memory

o Secondary memory

Register Memory

Register memory is the smallest and fastest memory in a computer. It is not a part  of the main memory and is located in the CPU in the form of registers, which are the  smallest data holding elements. A register temporarily holds frequently used data,  instructions, and memory address that are to be used by CPU. They hold instructions that  are currently processed by the CPU. All data is required to pass through registers before it  can be processed. So, they are used by CPU to process the data entered by the users.

Registers hold a small amount of data around 32 bits to 64 bits. The speed of a CPU  depends on the number and size (no. of bits) of registers that are built into the CPU.  Registers can be of different types based on their uses. Some of the widely used Registers  include Accumulator or AC, Data Register or DR, the Address Register or AR, Program  Counter (PC), I/O Address Register, and more.

Types and Functions of Computer Registers:

o Data Register: It is a 16-bit register, which is used to store operands (variables) to  be operated by the processor. It temporarily stores data, which is being  transmitted to or received from a peripheral device.

o Program Counter (PC): It holds the address of the memory location of the next  instruction, which is to be fetched after the current instruction is completed. So, it  is used to maintain the path of execution of the different programs and thus  executes the programs one by one, when the previous instruction gets completed.

o Instructor Register: It is a 16-bit register. It stores the instruction which is fetched  from the main memory. So, it is used to hold instruction codes, which are to be  executed. The Control Unit takes instruction from Instructor Register, then  decodes and executes it.

o Accumulator Register: It is a 16-bit register, which is used to store the results  produced by the system. For example, the results generated by CPU after the  processing are stored in the AC register.

o Address Register: It is a 12-bit register that stores the address of a memory  location where instructions or data is stored in the memory.

o I/O Address Register: Its job is to specify the address of a particular I/O device. o I/O Buffer Register: Its job is to exchange the data between an I/O module and the  CPU.

Cache Memory

Cache memory is a high-speed memory, which is small in size but faster than the  main memory (RAM). The CPU can access it more quickly than the primary memory. So,  it is used to synchronize with high-speed CPU and to improve its performance.

Cache memory can only be accessed by CPU. It can be a reserved part of the main  memory or a storage device outside the CPU. It holds the data and programs which are  frequently used by the CPU. So, it makes sure that the data is instantly available for CPU  whenever the CPU needs this data. In other words, if the CPU finds the required data or  instructions in the cache memory, it doesn't need to access the primary memory (RAM).  Thus, by acting as a buffer between RAM and CPU, it speeds up the system performance.

Types of Cache Memory:

L1: It is the first level of cache memory, which is called Level 1 cache or L1 cache.  In this type of cache memory, a small amount of memory is present inside the CPU  itself. If a CPU has four cores (quad core cpu), then each core will have its own level  1 cache. As this memory is present in the CPU, it can work at the same speed as of  the CPU. The size of this memory ranges from 2KB to 64 KB. The L1 cache further  has two types of caches: Instruction cache, which stores instructions required by  the CPU, and the data cache that stores the data required by the CPU.

L2: This cache is known as Level 2 cache or L2 cache. This level 2 cache may be  inside the CPU or outside the CPU. All the cores of a CPU can have their own  separate level 2 cache, or they can share one L2 cache among themselves. In case  it is outside the CPU, it is connected with the CPU with a very high-speed bus. The  memory size of this cache is in the range of 256 KB to the 512 KB. In terms of speed,  they are slower than the L1 cache.

L3: It is known as Level 3 cache or L3 cache. This cache is not present in all the  processors; some high-end processors may have this type of cache. This cache is  used to enhance the performance of Level 1 and Level 2 cache. It is located outside  the CPU and is shared by all the cores of a CPU. Its memory size ranges from 1 MB  to 8 MB. Although it is slower than L1 and L2 cache, it is faster than Random Access  Memory (RAM).

How does cache memory work with CPU?

When CPU needs the data, first of all, it looks inside the L1 cache. If it does  not find anything in L1, it looks inside the L2 cache. If again, it does not find the  data in L2 cache, it looks into the L3 cache. If data is found in the cache memory,  then it is known as a cache hit. On the contrary, if data is not found inside the  cache, it is called a cache miss.

If data is not available in any of the cache memories, it looks inside the  Random Access Memory (RAM). If RAM also does not have the data, then it will  get that data from the Hard Disk Drive.

So, when a computer is started for the first time, or an application is opened  for the first time, data is not available in cache memory or in RAM. In this case, the  CPU gets the data directly from the hard disk drive. Thereafter, when you start  your computer or open an application, CPU can get that data from cache memory  or RAM.

Primary Memory

Primary Memory is of two types: RAM and ROM.

RAM (Volatile Memory)

It is a volatile memory. It means it does not store data or instructions  permanently. When you switch on the computer the data and instructions from  the hard disk are stored in RAM.

CPU utilizes this data to perform the required tasks. As soon as you shut  down the computer the RAM loses all the data.

ROM (Non-volatile Memory)

It is a non-volatile memory. It means it does not lose its data or programs  that are written on it at the time of manufacture. So it is a permanent memory that  contains all important data and instructions needed to perform important tasks  like the boot process.

Secondary Memory

The secondary storage devices which are built into the computer or connected to  the computer are known as a secondary memory of the computer. It is also known as  external memory or auxiliary storage.

The secondary memory is accessed indirectly via input/output operations. It is non volatile, so permanently stores the data even when the computer is turned off or until this  data is overwritten or deleted. The CPU can't directly access the secondary memory. First,  the secondary memory data is transferred to primary memory then the CPU can access it.

Some of the secondary memory or storage devices are described below: 1) Hard Disk:

It is a rigid magnetic disc that is used to store data. It permanently stores  data and is located within a drive unit.

The hard disk is also known as a hard drive. It is a rigid magnetic disc that  stores data permanently, as it is a non-volatile storage device. The hard disk is  located within a drive unit on the computer's motherboard and comprises one or  more platters packed in an air-sealed casing. The data is written on the platters by  moving a magnetic head over the platters as they spin. The data stored on a  computer's hard drive generally includes the operating system, installed software,  and the user's files and programs, including pictures, music, videos, text  documents, etc.

Components of Hard Drive:

The main components of a hard drive include a head actuator, read/write  actuator arm, read/write head, platter, and spindle. A circuit board, which is called

the disk controller or interface board, is present on the back of a hard drive. It  allows the hard drive to communicate with the computer.

2) Solid-state Drive:

SSD (Solid State Drive) is also a non-volatile storage medium that is used  to hold and access data. Unlike a hard drive, it does not have moving components,  so it offers many advantages over SSD, such as faster access time, noiseless  operation, less power consumption, and more.

As the cost of SSD has come down, it has become an ideal replacement for  a standard hard drive in desktop and laptop computers. It is also suitable for  notebooks, and tablets that don't require lots of storage.

3) Pen drive:

Pen drive is a compact secondary storage device. It is also known as a USB  flash drive, thumb drive or a jump drive. It connects to a computer via a USB port.  It is commonly used to store and transfer data between computers. For example,  you can write a report using a computer and then copy or transfer it in the pen  drive. Later, you can connectthis pen drive to a computer to see or edit your report.  You can also store your important documents and pictures, music, videos in the  pen drive and keep it at a safe place.

Pen drive does not have movable parts; it comprises an integrated circuit  memory chip that stores the data. This chip is housed inside a plastic or aluminum  casing. The data storage capacity of the pen drive generally ranges from 2 GB to  128 GB. Furthermore, it is a plug and play device as you don't need additional  drives, software, or hardware to use it.

4) SD Card:

SD Card stands for Secure

Digital Card. It is most often used in

portable and mobile devices such as

smartphones and digital cameras.

You can remove it from your device

and see the things stored in it using a

computer with a card reader.

There are many memory chips inside

the SD card that store the data; it

does not have moving parts. SD cards are not created equal, so they may differ  from each other in terms of speed, physical sizes, and capacity. For example, standard SD cards, mini SD cards, and micro SD cards.

5) Compact Disk (CD):

Compact Disk is a portable secondary

storage device in the shape of a round

medium disk. It is made of polycarbonate

plastic. The concept of CD was co-developed

by Philips and Sony in 1982. The first CD was

created on 17 August 1982 at the workshop of

Philips in Germany.

In the beginning, it was used for storing and playing sound recordings, later  it was used for various purposes such as for storing documents, audio files, videos,  and other data like software programs in a CD.

Physical characteristics of a CD/ Structure of CD:

A standard CD is around 5 inches in diameter and 0.05 inches in thickness.  It is made of a clear polycarbonate plastic substrate, a reflective metallic layer, and  a clear coating of acrylic plastic. These thin circular layers are attached one on top  of another as described below:

o A polycarbonate disc layer at the bottom has the data encoded by creating  lands and pits.

o The polycarbonate disc layer is coated with a thin aluminum layer that  reflects the laser.

o The reflective aluminum layer is coated with a lacquer layer to prevent  oxidation in order to protect the below layers. It is generally spin coated  directly on the top of the reflective layer.

o The label print is applied on the lacquer layer, or artwork is screen printed  on the top of the disc on the lacquer layer by offset printing or screen  printing.

How Does a CD Work?

The data or information is stored or recorded or encoded in CD digitally  using a laser beam that etches tiny indentations or bumps on its surface. The bump

is called a pit, which represents the number 0. Space, where the bump is not  created, is called land, and it represents the number 1. Thus, the data is encoded  into a compact disc by creating pits (0) and lands (1). The CD players use laser  technology to read the optically recorded data.

6) DVD:

DVD is short for digital versatile disc or digital video

disc. It is a type of optical media used for storing

optical data. Although it has the same size as a CD,

its storage capacity is much more than a CD. So, it

is widely used for storing and viewing movies and to

distribute software programs as they are too large

to fit on a CD. DVD was co-developed by Sony,

Panasonic, Philips, and Toshiba in 1995.

Types of DVDs:

DVDs can be divided into three main categories which are as follows:

o DVD-ROM (Read-Only): These types of DVDs come with media already  recorded on them, such as movie dvds. As the name suggests, data on  these discs cannot be erased or added, so these discs are known as a read only or non-writable DVD.

o DVD-R (Writable): It allows you to record or write information to the DVD.  However, you can write information only once as it becomes a read-only  DVD once it is full.

o DVD-RW (Rewritable or Erasable): This type of discs can be erased,  written, or recorded multiple times.

Memory Units

Memory unit is the amount of data that can be stored in the storage unit in which storage  capacity is expressed in terms of bytes.

Following are the main memory storage units:

1. Bit (Binary Digit): The computer memory units start from bit. A bit is the smallest  memory unit to measure data stored in main memory and storage devices. A bit can  have only one binary value out of 0 and 1.

2. Nibble: A group of 4 bits is called nibble.

3. Byte: It is the fundamental unit to measure data. It contains 8 bits or is equal to 8 bits.  Thus a byte can represent 2*8 or 256 values.

4. Word: A computer word, like a byte, is a group of fixed number of bits processes as a  unit which varies from computer to computer but is fixed for each computer. The  length of a computer word is called word-size or word length and it may be as small as  8 bits or may be as long as 96 bits. A computer stores the information in the form of  computer words.

Few higher storage units are the following:

• Kilobyte (KB): 1 KB = 1024 bytes

• Megabyte (MB): 1 MB = 1024 KB

• Gigabyte (GB): 1 GB = 1024 MB

• Terabyte (TB): 1 TB = 1024 GB

• Petabyte (PT): 1 PB = 1024 TB

p